Switching the Transparency of Semiconductor Thin Films

Junjun Jia1, Yi Yin2

1 Faculty of Science and Engineering, Waseda University, Tokyo, JAPAN

2 State Key Laboratory of Electronic Thin Film and Integrated Device, University of Electronic Science and Technology of China, Chengdu, China

Email: jia@aoni.waseda.jp

When a semiconductor material is irradiated by an intense laser with excitation energy larger than its band gap, a large number of valence electrons can be excited into its conduction band. These excited electrons rapidly relax, and then transiently occupy at the bottom of the conduction band. Such transient occupation, named transient Pauli blocking effect [1, 2], reduces the material’s ability to absorb light, potentially enabling the switching from an opaque state to a transparent one on ultrafast timescales. This physics provides a foundation for designing ultrafast optical switching devices. Even more remarkably, when the laser is modulated at ultrahigh speeds, the effect can be dynamically controlled, unlocking new possibilities for advanced optical technologies [1].

In this talk, I will present recent progress in ultrafast optical switching in semiconductor films, with a particular focus on multivalley materials [1]. By irradiating the multivalley material Ge with an intense pulsed laser, ultrafast optical switching between transparency and opacity across a wide wavelength range is achieved. Femtosecond time-resolved transient transmission measurements revealed that this ultrafast optical switching occurs in both the Γ and L valleys, enabled by intravalley and intervalley scattering processes. Moreover, I also will discuss the shift in plasmon resonance wavelength in transparent conducting materials induced by intense laser irradiation, which offers an additional pathway for developing optical switching devices  [3]. These works are expected to open new doors for possible applications in optical multiband communications, optical computing, and beyond.

This work is supported by JSPS KAKENHI Grant-in-Aid for Scientific Research (B) (Grant No. 25K01862).

References:

[1] J. Jia, H. A. Almossalami, H. Ye, N. Yamada, T. Yagi, Phys. Rev. Applied, 23, 024060 (2025).

[2] J. Jia, T. Yagi, M. Mizutani, N. Yamada, and T. Makimoto, J. Appl. Phys. 132, 165702 (2022.)

[3] Q. Hu, J. Jia, et al., ACS Photonics, 10, 3612 (2023).