Bioinspired Flexible Intelligent Interface Materials and Systems for Health Monitoring

Beibei Shao1,2

1State Key Laboratory of Bioinspired Interfacial Materials Science, Institute of Functional Nano & Soft Materials (FUNSOM), Soochow University, Suzhou, China

2Department of Materials Science and Engineering, National University of Singapore, Singapore 117574, Singapore

E-mail: bbshao@suda.edu.cn

The development of intelligent, soft electronics that conform to the human body is essential for next-generation health monitoring. Here, we present a bioinspired materials and systems framework that integrates multiscale interfacial design, self-powered sensing, and AI-assisted signal processing to enable real-time, noninvasive physiological monitoring[1-10] We fabricate flexible and stretchable self-powered devices that intimately adhere to the skin. These systems integrate humidity-responsive ionotronic layers, triboelectric sensors, and soft nanogenerators, enabling continuous and autonomous monitoring of skin hydration, body motion, and acoustic signals. The devices maintain stable performance under mechanical deformation and high humidity, ensuring reliable operation in wearable environments. To realize intelligent functionality, we incorporate lightweight deep learning models capable of decoding biosignals and generating responsive feedback. The system accurately predicts dehydration risks, detects physical fatigue, and classifies voice commands, supporting closed-loop health management. At the materials level, we design transparent dual-ion conductive hydrogels via molecular assembly and interfacial functionalization. These hydrogels mimic biological ion channels, facilitating directional ion transport and efficient hydrovoltaic energy harvesting from ambient moisture and perspiration. Our work presents a unified strategy for developing bioinspired, flexible, and intelligent electronic systems, bridging materials science, energy harvesting, and artificial intelligence to advance personalized, adaptive, and autonomous health monitoring.

References

[1] Shao B.#, Lu M. H.#, Wu T. C., Peng W. C., Ko T. Y., Hsiao Y. C., Chen J. Y., Sun B., Liu R., Lai Y. C.* Large-area, untethered, metamorphic, and omnidirectionally-stretchable multiplexing self-powered triboelectric skins. Nature Communications, 2024, 15, 1238

[2] Shao B.#, Lu T. C.#, Lu M. H., Chen Y. T., Wu T. C., Peng W. C., Ko T. Y., Chen J. Y., Sun B., Chen C. Y., Liu R., Hsu F. C., Lai Y. C.* Efficient permeable monolithic hybrid tribo-piezo-electromagnetic nanogenerator based on topological-insulator-composite. Advanced Materials, 2024, 2408936

[3] Wang Y., Yuan X., Ni K., Song Y., Li X., Zeng X., Shao B.*, Sun B.* 3D dendritic hierarchically gradient nanoflowers in situ grown on conductive substrates for efficient hydrovoltaic power generation. Energy & Environmental Science, 2024, 17, 4780

[4] Duan W.#, Shao B.#, Wang Z., Ni K., Liu S., Yuan X., Wang Y., Sun B., Zhang X., Liu R.* Silicon nanowire/ionic hydrogel-based hybrid moist-electric generators with enhanced voltage output and operational stability. Energy & Environmental Science, 2024, 17, 3788

[5] Shao B., Song Y., Song Z., Wang Y., Wang Y.*, Liu R.*, Sun B.* Electricity generation from phase transitions between liquid and gaseous water. Advanced Energy Materials, 2023, 13, 2204091

[6] Shao B., Song Z., Chen X., Wu Y., Li Y., Song C., Yang F., Song T., Wang Y.*, Lee S. T.*, Sun B.* Bioinspired hierarchical nanofabric electrode for silicon hydrovoltaic device with record power output. ACS Nano, 2021, 5, 7472

[7] Song Y., Song Z., Jiang C., Xing C., Zeng X., Zhang Z., Chen Z., Song T., Shao B.*, Wang Y.*, Sun B.* Ferroelectric layer-assisted asymmetric heterojunction boosts power output in silicon hydrovoltaic device. Advanced Energy Materials, 2023, 2302765

[8] Zeng X., Ding Y., Yuan X., Jiang C., Wang Y., Song Y., Li Y., Shao B.*, Wang Y.*, Sun B.* Photo-mediated cascade growth of Ag nanocrystals in flow reactor for high-performance flexible transparent electrodes. Advanced Functional Materials, 2023, 2309730

[9] Shao B., Xing C., Song Y., Jiang C., Bai G., Song T., Liu R.*, Wang Y.*, Sun B.* Boosting electrical output of nanostructured silicon hydrovoltaic device via cobalt oxide enabled electrode surface contact. Nano Energy, 2023, 106, 108081

[10] Shao B., Wu Y., Song Z., Yang H., Chen X., Zou Y., Zang J., Yang F., Song T., Wang Y.*, Shao M.*, Sun B.* Freestanding silicon nanowires mesh for efficient electricity generation from evaporation-induced water capillary flow. Nano Energy, 2022, 94, 106917